UART – APB design Verification plan

Title
UART – APB design Verification plan

File Name
cmsdk_apb_uart.v

Author
Raju Gupta

Created On
04-08-2025

- DUT Overview

## Description

This document describes the implementation and verification of a simplified UART RTL design based on the AMBA APB interface. The module cmsdk_apb_uart implements a serial communication interface that supports:

- 8-bit UART transmission and reception

- Programmable baud rate via a 20-bit baud divider register BAUDIV

- Generation and clearing of multiple interrupts: TXINT, RXINT, TXOVRINT, RXOVRINT, and UARTINT

- High-speed test mode operation for simulation acceleration

- Compliance with APB3 register protocol for integration in AMBA-based SoCs

- TX and RX logic managed by FSMs with tick-based state transitions

| ### 5 ### 2. Interface Overview #### APB Interface Signals Inputs: - PCLK: Clock signal for APB operations - PRESETn: Active-low reset - PSEL: APB peripheral select - PENABLE: APB enable phase - PWRITE: Indicates write (1) or read (0) - PADDR[11:2]: Register address (word-aligned) - PWDATA[31:0]: Write data from APB master Outputs: - PRDATA[31:0]: Data read from UART registers #### UART Serial Interface Signals Input: - RXD: UART receive data line (serial input) Outputs: - TXD: UART transmit data line (serial output) - TXINT: Transmit interrupt (TX buffer ready) - RXINT: Receive interrupt (byte received) - TXOVRINT: TX overrun interrupt - RXOVRINT: RX overrun interrupt - UARTINT: Combined interrupt signal (logical OR of all UART interrupts) Note: RXD input is passed through a double flip-flop synchronizer in the DUT. Monitors should account for one PCLK cycle latency when validating RX state transitions. ## 3. Internal Architecture ### FSMs Two finite state machines (FSMs) control UART transmission and reception: - Transmit FSM - States: IDLE, START, DATA0–DATA7, STOP - Transitions: Driven by TX enable, baud tick, and TX buffer status - Receive FSM - States: IDLE, START DETECT, DATA0–DATA7, STOP - Transitions: Based on RXD edge detection and tick count sampling ### Registers and Logic - reg_tx_buf, reg_rx_buf: 8-bit transmit and receive buffers - reg_ctrl[6:0]: Control register (TX/RX enable, INT enable, test mode) - reg_status[3:0]: Status register (TXFULL, RXFULL, overruns) - reg_baud_div[19:0]: Baud rate divider register - reg_baud_cntr_i/f: Integer and fractional tick counters - tx_shift_buf: 8-bit shift register for serial TX - rx_sample_buf: Captures incoming RXD bits for assembly - tx_state, rx_state: FSM state registers - reg_tx_overrun, reg_rx_overrun: Error flags for TX and RX buffer overruns ## Protocol Compliance ### Supported Features - Full-duplex 8-bit UART communication (TX and RX) - Standard UART 1 start bit + 8 data bits + 1 stop bit format - Configurable baud rate using 20-bit BAUDDIV register - APB3-compliant register interface - Individual interrupt generation: TXINT, RXINT, TXOVRINT, RXOVRINT - Combined interrupt output: UARTINT - High-speed test mode for simulation acceleration ### Unsupported Features - Parity bit and framing configuration - Hardware flow control (RTS/CTS) - Break condition and loopback modes - FIFO buffering or DMA integration (not implemented in RTL) |  |
| --- | --- |

4. Verification Objectives

- Verify full APB register access including read/write/INTCLEAR behavior

- Validate UART TX and RX logic for both functional and timing correctness

- Verify UART interrupt logic under normal and corner-case scenarios

- Test overruns and framing error conditions with assertions

- Validate baud tick timing under normal and high-speed mode

- Confirm FSM transitions and state legality in TX and RX datapaths

- Achieve 100% functional, code, and FSM coverage

- Integrate assertions for protocol rule checks

- Ensure reusability in subsystem/full-chip environment

- Validate baud rate configuration using BAUDDIV = PCLK / Baud

- Ensure system can handle UART interrupt timing at high speeds (e.g., 115200 baud)

5. Verification Environment Components

- APB Agent: Drives and sequences APB read/write transactions to configure the UART registers.

- UART Driver: Sends serial data to the DUT by driving RXD with valid UART frames.

- UART Monitor: Observes TXD to reconstruct and check transmitted UART bytes.

- Register Model (RAL): Maintains a mirrored view of UART registers and verifies APB accesses.

- Scoreboard: Compares expected and actual data from TXD, RX buffer, and registers.

- Virtual Sequencer: Coordinates APB and UART sequences to ensure proper flow.

- Coverage Collector: Tracks functional, code, FSM, and toggle coverage for completeness.

- Clock/Reset Generator: Provides PCLK and PRESETn signals to the DUT and interfaces.

6. Functional Testcases

| TC No. | Testcase Name | Description |
| --- | --- | --- |
| TC01 | UART Soft Reset | Apply reset and check all internal registers and outputs |
| TC02 | Basic TX Transmission | Write 1 byte to TX buffer. Ensure TXD outputs start bit, 8 data bits (LSB first), and stop bit. |
| TC03 | Basic RX Reception | Send a UART frame (start + 8 data bits + stop) on RXD and verify RX buffer gets updated. |
| TC04 | Transmit Buffer Full Handling | Write multiple bytes to TX buffer until tx_buf_full is asserted. |
| TC05 | Receive Buffer Full Handling | Send two back-to-back UART frames without reading the first. Verify rx_buf_full and overrun. |
| TC06 | TX Overrun Interrupt | Write to TX buffer when it’s full. Confirm TXOVRINT is raised. |
| TC07 | RX Overrun Interrupt | Simulate RX overrun by sending two frames quickly. Verify RXOVRINT. |
| TC08 | Interrupt Enable/Disable | Enable/disable TXINT and RXINT via control reg. Verify only enabled interrupts are triggered. |
| TC09 | INTCLEAR Operation | Write to INTCLEAR register. Confirm TXINT/RXINT/OVRINT are cleared. |
| TC10 | Status Register Read | Check status register bits (tx_buf_full, rx_buf_full, etc.) match internal states. |
| TC11 | Baud Rate Programming (Integer only) | Write a valid baud divider value (≥ 16) and verify reload_i pulses every N cycles. |
| TC12 | Baud Rate Less than 16 (Illegal) | Write divider < 16 and ensure assertion or error response (as per OVL). |
| TC13 | Baud Rate Change During Idle | Change baud divider when TX is idle. Ensure proper reload without glitch. |
| TC14 | Baud Rate Change During TX | Try to change baud divider mid-transmission. Ensure it is blocked or handled cleanly. |
| TC15 | Read RX Buffer | Send a UART byte and read from RX register. Ensure correct value and rx_buf_full clears. |
| TC16 | TXINT Trigger Test | Fill TX buffer, then check TXINT is raised only when the buffer is emptied (as per spec). |
| TC17 | RXINT Trigger Test | Send data and verify RXINT gets raised after valid byte is received. |
| TC18 | High-Speed Test Mode | Enable high-speed mode (reg_ctrl[6]) and verify TX state changes in 1 cycle. |
| TC19 | Illegal TX/RX States Check (FSM) | Force illegal FSM states (rx_state=0xB-0xF, tx_state=0xC-0xF) and verify assertions. |
| TC20 | INTCLEAR Byte-Wise Write | Write only bit [1] or bit [0] of INTCLEAR and confirm selective interrupt clearing. |

7. Assertions 
Description: Ensures correct UART protocol operation using SystemVerilog Assertions (SVA)

- TXD Idle Check

  - Property Name: txd_idle

  - Description: TXD must be high during idle and after reset.

- RX Start Bit Detect

  - Property Name: rxd_startbit_detect

  - Description: RXD must be low to initiate start bit detection.

- TX Tick Timing

  - Property Name: tx_bit_tick_count

  - Description: Each TX bit must last exactly 16 baud ticks (unless in high-speed mode).

- RXINT Assertion

  - Property Name: rxint_enable_check

  - Description: RXINT must assert only when RX is enabled and byte is received.

- BAUDDIV Minimum Check

  - Property Name: baud_div_min_check

  - Description: BAUDDIV must be ≥ 16 when UART is active.

## 8. Coverage

### Functional Coverage

The verification plan includes coverpoints for:

- Baud rate divider (BAUDDIV) values, including valid range and illegal configurations.

- TX buffer operations, including data values, full conditions, and TXINT/TXOVRINT generation.

- RX behavior: start bit detection, valid data reception, stop bit, framing errors, and RXINT/RXOVRINT.

- Control register (reg_ctrl) fields: TX/RX enable, interrupt enable, and test mode activation.

- INTCLEAR register access: individual interrupt clear actions.

- Status register flags: TXFULL, RXFULL, and overrun flags read by software.

### Code Coverage

- Line and condition coverage of all RTL modules.

- FSM state and transition coverage for TX and RX state machines.

- Toggle coverage of control and data signals (TXD, RXD, INTs, buffer flags).

9. Additional parameters

- PCLK = 50 MHz, Baud Rate = 115200 bps

- BAUDDIV = 27, calculated as:

      - BAUDDIV = PCLK / (16 × Baud Rate)

      - BAUDDIV = 50,000,000 / (16 × 115200) ≈ 27

- 1 character = 10 bits → ~4340 PCLK cycles per character